Mini Analysis Instructions

In Clampfit

- Open all the recordings that contain spontaneous currents from one cell.  Check the baseline recording to make sure it is 5 minutes (60 sweeps) long. If there are more than 60 sweeps, right click on the recording window and click Select sweeps.  Select the last 60 sweeps and click Ok.  Save the baseline recording as a new file with the name Recording ID_last 5 minutes.

- Concatenate the recordings (including either the regular baseline recording or the modified baseline recording, if necessary) into one file.  Click Analyze → Concatenate Files and then select all of the recordings from that cell.

- Remove the stimulation artifacts and evoked currents.

  - Put cursors 1 and 2 around a small baseline region before the first stimulation artifact and put cursors 3 and 4 around a small baseline region before the second stimulation artifact.

  - Put cursors 5 and 6 around the first stimulation artifact/current and repeat with cursors 7 and 8 around the second stimulation artifact/current.

  - Select View → Window Defaults → Save selected analysis window settings as default.  This step is only necessary when you are concatenating your first file.

  - Click Analyze → Force Values and set the baseline value as the mean between cursors 1 and 2 and the region to force as the region between cursors 5 and 6.  ALL VISBILE SIGNALS AND ALL VISBLE TRACES!

  - Click Force Values.  Repeat with cursors 3 and 4 as the baseline and 7 and 8 as the region to force.  ALL VISBILE SIGNALS AND ALL VISBLE TRACES!

- Save the file as ABF 1.8 (integer) (*ABF) and label the file “Concatenate-xx” (with xx being the cell ID).

In Mini Analysis: NO YOU DON’T WANT TO REGISTER

- Select file to open (if you recently saved files to the folder of interest, you may need to refresh that folder for the new files to appear in Mini Analysis).

- Enter channel number: 0.

- Scroll through the first ~1-2 minutes of the recording and look for changes in the thickness of the baseline.  If the baseline appears to be consistent, zoom into the recording by entering 2 blocks at the top of the screen.

- START AT BEGINNING. Look for a completely flat region of the recording.  There should be no events in your window.  Use pink cursors to scroll L/R (you can also use “D” on the keyboard to scroll to the right) until you find a flat section of the recording (but not a window that includes the flattened evoked currents).

- Select Noise Analysis → Measure RMS noise.

  - Enter block number corresponding to the current block region (i.e. 2).

  - A window will open with the RMS noise value.  Record this value in your lab book (ideally should be 1-2ish). Select “yes” in response to “Would you like to set the detection threshold based on this value?”

  - Enter 5 for the multiplication factor for threshold (should ideally be around 5-10).

- The baseline threshold is now set.  Record the threshold value in your lab book.

- Zoom out to 150 blocks.

- Click K (or A) to go to the beginning of the recording.

- Click Non-stop Analysis.  Wait for the analysis to finish.

- Use the |< cursor to navigate to the start of the time window.

- Click Analyze this Window and repeat this step for the second window.  (because it didn’t do the first window automatically)

- Select “Events” → Group by Criteria Search.

  - For the rise (10-90) ms, enter minimum: 0.3 and maximum: 4.

  - Mark the results as Group 1.

  - Mark the rest as Group 255 (check box).

  - Click Start Search.

- Carefully scan through the recording to ensure that the software is not recognizing artifacts or noise as currents and to ensure that it is recognizing all spontaneous currents.  Depending on the quality of the recording, you may need to zoom in to ~200 blocks or zoom out to ~1000 blocks.

  - Look for changes in the thickness of the baseline.  If it changes significantly, you may need to recalculate the RMS noise after the baseline changes.

  - Look for spontaneous currents that were missed.

    - To mark a spontaneous current that was not detected, LEFT CLICK TO HIGHLIGHT IT (X), right click on the current and click Mark as Group and select 1

  - Look for events that were incorrectly detected

    - To delete a spontaneous current that was incorrectly detected, right click on the current and select Delete (or unmark from group)

- Select “Events” → Delete All Rejected Events.  Only true spontaneous currents should now be marked with a 1.

- Click Plot → Frequency Histogram → Time → Group 1 → Bin Size 5.  Check to make sure the graph doesn’t look strange (i.e. there is a large, unexpected change in the frequency of the currents).

- Click Graph and Data → Copy Graph Values to Clipboard.  Paste the values in an excel sheet.

- Click Plot → Running Average Histogram → Amplitude → Bin size 5.

- Click Graph and Data → Copy Graph Values to Clipboard.  Paste the values in an excel sheet.

Excel sheet eventually:

“letter”, ID, “amplitude:, SE, “time”, “num_events”